GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLIES LIMITED

SUPER-MINI HOME SPECIFICATIONS

JUNE 1, 2021

PAGE 1 OF 1

MATERIAL SUPPLIED IN SUPER-MINI HOME SHELL PACKAGE

(STAGE 1) PAGE 1 OF SPECIFICATIONS MINI-HOME SHELL PACKAGE

HOUSE PLANS:	A COMPLETE SET OF PLANS SHOWING FOUNDATION, FLOOR LAYOUTS, ELEVATIONS, DETAIL ETC. ARE SUPPLIED.

SILL PLATE:	2X6 WOLMANIZED SILL PLATE IS SUPPLIED.

FLOOR COMPONENTS:	MATERIAL FOR LAMINATED 8X 8 POSTS ARE SUPPLIED. BEAM STOCK 2” X 8” OR 2” X 10” PER PLAN. 1” X 3” BRIDGING IS SUPPLIED. 5/8 T & G FLOOR OSB SHEATHING IS SUPPLIED.

EXTERIOR WALLS:	2X6 PLATES, SINGLE BOTTOM, DOUBLE TOP. 2X6 STUDS AT 16 O.C. ARE SUPPLIED. 15/32 OSB SHEATHING IS SUPPLIED FOR WALLS AND GABLE ENDS.

INTERIOR PARTITIONS:	2” X 4”  PLATES, SINGLE BOTTOM, DOUBLE TOP. 2” X 4” STUDS AT 16” O.C. ARE SUPPLIED.

ROOF/CEILING FRAMING:	ROOF TRUSSES ARE SUPPLIED FOR GABLE ROOFS AT 24” O.C. GABLE END TRUSSES ARE SUPPLIED. 1X4 STRAPPING IS SUPPLIED FOR CEILINGS.

ROOF SHEATHING:	15/32 OSB SHEATHING IS SUPPLIED FOR ROOF AND GABLE ENDS.

ROOF SHINGLES:	15# FELT PAPER, WEATHERSHIELD MATERIAL, 20 YEAR ASPHALT SHINGLES AND ROOFING CEMENT ARE SUPPLIED.

EXTERIOR DOORS:	PRE-HUNG STEEL ENTRANCE DOORS WITH HARDWARE ARE SUPPLIED.

WINDOWS:	VINYL PICTURE WINDOWS PER PLAN.  VERTICAL SINGLE HUNG  WINDOWS ARE SUPPLIED UNLESS OTHERWISE SPECIFIED. UNITS ARE EQUIPPED WITH SEALED GLASS AND SCREENS. MUTTON BARS  ARE SUPPLIED.

FLASHING:	ZINC FLASHING IS SUPPLIED FOR ALL WINDOWS AND EXTERIOR DOORS.

SIDING:	VINYL SIDING AND VINYL SOFFIT MATERIALS IS SUPPLIED. ARE SUPPLIED.

INSULATION:	R-24 FIBREGLAS INSULATION IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS. R-50 IS FIBREGLAS INSULATION IS SUPPLIED FOR CEILING AREAS. 6 MIL POLYETHELENE VAPOUR BARRIER, ADHESIVES AND TAPE ARE SUPPLIED.

WALLBOARD:	1/2 GYPSUM BOARD IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS, INTERIOR PARTITIONS, CEILINGS IN FINISHED AREAS. CRACKFILLING MATERIAL AND TAPES ARE SUPPLIED.

WITNESS:___________________________	BUYER:___________________________

WITNESS:___________________________	BUYER:___________________________