GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLIES LIMITED

AFFORDABLE HOME SPECIFICATIONS

JUNE 1, 2021

PAGE 1 OF 1

MATERIAL SUPPLIED IN AFFORDABLE HOME SHELL PACKAGE

(STAGE 1) PAGE 1 0F SPECIFICATIONS - AFFORDABLE HOME SHELL PACKAGE

HOUSE PLANS:	A COMPLETE SET OF PLANS SHOWING FOUNDATION, FLOOR LAYOUTS, ELEVATIONS, DETAIL ETC. ARE SUPPLIED.

SILL PLATE:	2” X 6” WOLMANIZED SILL PLATE IS SUPPLIED.

FLOOR COMPONENTS:	MATERIAL FOR LAMINATED 8X 8 POSTS ARE SUPPLIED. BEAM STOCK 2” X 8“ OR 2” X 10” PER PLAN. 1” X 3” BRIDGING IS SUPPLIED. 5/8 OSB T & G FLOOR SHEATHING IS SUPPLIED.

EXTERIOR WALLS:	2” X 6” PLATES, SINGLE BOTTOM, DOUBLE TOP. 2” X 6” STUDS AT 16” O.C. ARE SUPPLIED.15/32 OSB SHEATHING IS SUPPLIED FOR WALLS AND GABLE ENDS.

INTERIOR PARTITIONS:	2” X 4” PLATES, SINGLE BOTTOM, DOUBLE TOP. 2” X 4” STUDS AT 16” O.C. ARE SUPPLIED.

ROOF/CEILING FRAMING:	ROOF TRUSSES ARE SUPPLIED FOR HIP AND GABLE ROOFS AT 24” O.C. GABLE END TRUSSES ARE SUPPLIED. 1” X 4”  STRAPPING IS SUPPLIED FOR CEILINGS.

ROOF SHEATHING:	15/32 OSB SHEATHING IS SUPPLIED FOR ROOF AND GABLE END.

ROOF SHINGLES:	15# FELT PAPER, WEATHERSHIELD MATERIAL, 25 YEAR ASPHALT SHINGLES AND ROOFING CEMENT ARE SUPPLIED.

EXTERIOR DOORS:	PRE-HUNG STEEL ENTRANCE DOORS WITH HARDWARE ARE SUPPLIED.

WINDOWS:	VINYL PICTURE WINDOWS PER PLAN.  VERTICAL SINGLE HUNG OR HORIZONTAL SLIDING WINDOWS ARE SUPPLIED UNLESS OTHERWISE SPECIFIED IN SOME MODELS. UNITS ARE EQUIPPED WITH SEALED GLASS AND SCREENS.  MUTTON BARS ARE SUPPLIED.

FLASHING:	ZINC FLASHING IS SUPPLIED FOR ALL WINDOWS AND EXTERIOR DOORS.

SIDING/SOFFIT:	VINYL SIDING AND SOFFIT MATERIAL IS SUPPLIED FOR EXTERIOR WALLS AND GABLE ENDS.

INSULATION:	R-24 FIBERGLASS INSULATION IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS. R-40 FIBERGLASS INSULATION IS SUPPLIED FOR CEILING AREAS. 6 MIL POLYETHYLENE VAPOUR BARRIER, ADHESIVES AND TAPE ARE SUPPLIED.

WALLBOARD:	1/2@ GYPSUM BOARD IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS, INTERIOR PARTITIONS, CEILINGS IN FINISHED AREAS. CRACK FILLING MATERIAL AND TAPES ARE SUPPLIED.

WITNESS:___________________________	BUYER:___________________________

WITNESS:___________________________	BUYER:___________________________