GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLIES LIMITED

JUNE 1, 2021

PAGE 1 OF 1

MATERIAL SUPPLIED IN CUSTOM HOME SHELL PACKAGE

(STAGE 1)-PAGE 1 OF SPECIFICATIONS)-CUSTOM HOME SHELL PACKAGE

HOUSE PLANS:	A COMPLETE SET OF PLANS SHOWING FOUNDATION, FLOOR LAYOUTS, ELEVATIONS, DETAIL ETC. ARE SUPPLIED.

BASEMENT WINDOWS:	VINYL BASEMENT WINDOWS ARE SUPPLIED. SPLIT ENTRY, SPLIT-LEVEL AND TWO STOREY MODELS ARE SUPPLIED WITH VINYL SINGLE HUNG WINDOWS UNLESS OTHERWISE SPECIFIED.

SILL PLATE:	2” X 6”  WOLMANIZED  SILL PLATE IS SUPPLIED.

FLOOR COMPONENTS:	ADJUSTABLE STEEL JACK POSTS ARE SUPPLIED FOR BEAM SUPPORT. BEAM STOCK 8” X 8” OR 8” X 10” PER PLAN.  FLOOR JOISTS SUPPLIED ARE 2” X  8” OR 2” X 10” PER PLAN. 1” X 3” BRIDGING IS SUPPLIED. 5/8 OSB T & G FLOOR SHEATHING IS SUPPLIED.

EXTERIOR WALLS:	2” X 6” PLATES, SINGLE BOTTOM, DOUBLE TOP. 2” X 6” STUDS AT 16” O.C. ARE SUPPLIED.15/32 OSB SHEATHING IS SUPPLIED FOR WALLS AND GABLE ENDS.

INTERIOR PARTITIONS:	2 X 4 PLATES, SINGLE BOTTOM, DOUBLE TOP. 2 X 4 STUDS AT 16" O.C. ARE SUPPLIED.

ROOF/CEILING FRAMING:	ROOF TRUSSES ARE SUPPLIED FOR HIP AND GABLE ROOFS AT 24” O.C. GABLE END TRUSSES ARE SUPPLIED PER PLAN. 1” X 4” STRAPPING IS SUPPLIED FOR CEILINGS.

ROOF SHEATHING:	15/32 OSB SHEATHING FOR ROOF AND GABLE ENDS.

ROOF SHINGLES:	15# FELT PAPER, WEATHERSHIELD MATERIAL, 25 YEAR ASPHALT SHINGLES AND ROOFING CEMENT ARE SUPPLIED.CEMENT IS SUPPLIED.

EXTERIOR DOORS:	PRE-HUNG STEEL ENTRANCE DOORS WITH HARDWARE ARE SUPPLIED.

WINDOWS:	VINYL SINLE HUNG WINDOWS ARE SUPPLIED UNLESS OTHERWISE SPECIFIED MODELS. UNITS ARE EQUIPPED WITH SEALED GLASS AND SCREENS. CENTER OF PICTURE WINDOWS ARE FIXED UNITS.  WINDOWS ON FRONT ELEVATION ARE SUPPLIED WITH SHUTTERS. MUTTON BARS ARE SUPPLIED.

FLASHING:	ZINC FLASHING IS SUPPLIED FOR ALL WINDOWS AND EXTERIOR DOORS.

SIDING/SOFFIT:	VINYL SIDING IS SUPPLIED. GABLE TRIM AND VINYL SOFFIT IS SUPPLIED.

EAVESTROUGHS:	VINYL EAVESTROUGH MATERIAL IS SUPPLIED.

INSULATION:	R-24 FIBREGLAS INSULATION IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS. R-40 FIBREGLAS INSULATION IS SUPPLIED FOR CEILING AREAS. 6 MIL  POLYETHELENE VAPOUR BARRIER, ADHESIVES AND TAPE ARE SUPPLIED.

WALLBOARD: 	GYPSUM BOARD IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS, INTERIOR PARTITIONS, CEILINGS IN FINISHED AREAS AND TO COVER WOOD FRAME PONY WALLS IN THE BASEMENT AREA IF APPLICABLE.  CRACKFILLING MATERIAL AND TAPE ARE SUPPLIED.

WITNESS:___________________________	BUYER:___________________________

WITNESS:___________________________	BUYER:___________________________