GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLIES LIMITED

JUNE 1, 2021

PAGE 1 OF 3

MATERIAL SUPPLIED IN CUSTOM HOME SHELL PACKAGE

(STAGE 1)-PAGE 1 OF SPECIFICATIONS)-CUSTOM HOME SHELL PACKAGE

HOUSE PLANS:	A COMPLETE SET OF PLANS SHOWING FOUNDATION, FLOOR LAYOUTS, ELEVATIONS, DETAIL ETC. ARE SUPPLIED.

BASEMENT WINDOWS:	VINYL BASEMENT WINDOWS ARE SUPPLIED. SPLIT ENTRY, SPLIT-LEVEL AND TWO STOREY MODELS ARE SUPPLIED WITH VINYL SINGLE HUNG WINDOWS UNLESS OTHERWISE SPECIFIED.

SILL PLATE:	2” X 6”  WOLMANIZED  SILL PLATE IS SUPPLIED.

FLOOR COMPONENTS:	ADJUSTABLE STEEL JACK POSTS ARE SUPPLIED FOR BEAM SUPPORT. BEAM STOCK 8” X 8” OR 8” X 10” PER PLAN.  FLOOR JOISTS SUPPLIED ARE 2” X  8” OR 2” X 10” PER PLAN. 1” X 3” BRIDGING IS SUPPLIED. 5/8 OSB T & G FLOOR SHEATHING IS SUPPLIED.

EXTERIOR WALLS:	2” X 6” PLATES, SINGLE BOTTOM, DOUBLE TOP. 2” X 6” STUDS AT 16” O.C. ARE SUPPLIED.15/32 OSB SHEATHING IS SUPPLIED FOR WALLS AND GABLE ENDS.

INTERIOR PARTITIONS:	2 X 4 PLATES, SINGLE BOTTOM, DOUBLE TOP. 2 X 4 STUDS AT 16" O.C. ARE SUPPLIED.

ROOF/CEILING FRAMING:	ROOF TRUSSES ARE SUPPLIED FOR HIP AND GABLE ROOFS AT 24” O.C. GABLE END TRUSSES ARE SUPPLIED PER PLAN. 1” X 4” STRAPPING IS SUPPLIED FOR CEILINGS.

ROOF SHEATHING:	15/32 OSB SHEATHING FOR ROOF AND GABLE ENDS.

ROOF SHINGLES:	15# FELT PAPER, WEATHERSHIELD MATERIAL, 25 YEAR ASPHALT SHINGLES AND ROOFING CEMENT ARE SUPPLIED.CEMENT IS SUPPLIED.

EXTERIOR DOORS:	PRE-HUNG STEEL ENTRANCE DOORS WITH HARDWARE ARE SUPPLIED.

WINDOWS:	VINYL SINLE HUNG WINDOWS ARE SUPPLIED UNLESS OTHERWISE SPECIFIED MODELS. UNITS ARE EQUIPPED WITH SEALED GLASS AND SCREENS. CENTER OF PICTURE WINDOWS ARE FIXED UNITS.  WINDOWS ON FRONT ELEVATION ARE SUPPLIED WITH SHUTTERS. MUTTON BARS ARE SUPPLIED.

FLASHING:	ZINC FLASHING IS SUPPLIED FOR ALL WINDOWS AND EXTERIOR DOORS.

SIDING/SOFFIT:	VINYL SIDING IS SUPPLIED. GABLE TRIM AND VINYL SOFFIT IS SUPPLIED.

EAVESTROUGHS:	VINYL EAVESTROUGH MATERIAL IS SUPPLIED.

INSULATION:	R-24 FIBREGLAS INSULATION IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS. R-40 FIBREGLAS INSULATION IS SUPPLIED FOR CEILING AREAS. 6 MIL  POLYETHELENE VAPOUR BARRIER, ADHESIVES AND TAPE ARE SUPPLIED.

WALLBOARD: 	GYPSUM BOARD IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS, INTERIOR PARTITIONS, CEILINGS IN FINISHED AREAS AND TO COVER WOOD FRAME PONY WALLS IN THE BASEMENT AREA IF APPLICABLE.  CRACKFILLING MATERIAL AND TAPE ARE SUPPLIED.

WITNESS:___________________________	BUYER:___________________________

WITNESS:___________________________	BUYER:___________________________

GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLIES LIMITED

JUNE 1, 2021

PAGE 2 OF 3

ADDITIONAL MATERIALS SUPPLIED IN THE CUSTOM HOME PACKAGE

(STAGE 2)- PAGES 1 AND 2 OF SPECIFICATIONS-CUSTOM HOME CUSTOM PACKAGE

BASEMENT/MAIN STAIRS:	2 X 10 STRINGERS, SPRUCE TREADS, PINE RISERS ARE SUPPLIED.

BASEBOARD:	COLONIAL BASEBOARD MATERIAL IS SUPPLIED.

WINDOW TRIM:	COLONIAL WINDOW TRIM IS SUPPLIED.

EXTERIOR DOOR TRIM:	COLONIAL EXTERIOR DOOR TRIM IS SUPPLIED.

INSIDE DOORS:	#800 COLONIAL SLAB DOORS ARE SUPPLIED.  COLONIAL TRIM IS SUPPLIED FOR SLAB TYPE DOORS.  #800 BI-FOLD DOORS ARE SUPPLIED FOR CLOSET AREAS AND WHERE INDICATED ON THE PLANS. THE BI-FOLD DOOR OPENINGS ARE THE MODERN TYPE WITHOUT TRIM.

CLOSETS:	CLOSET RODS AND PINE SHELVING ARE SUPPLIED FOR ALL LINEN AND CLOTHES CLOSETS.

UNDERLAY:	UNDERLAY IS SUPPLIED FOR APPLICATION OVER WOODEN SUB-FLOORS EXCEPT 10MM MIRANDA BOARD IS SUPPLIED FOR AREAS WHERE VINYL FLOORING WILL BE INSTALLED.

BATHROOM ACCESSORIES: SOAP AND GRAB, TOILET TISSUE HOLDER, SOAP AND GRAB AND SHOWER BAR ARE SUPPLIED.

KITCHEN CABINETS:	OAK, BIRCH, PINE OR SPRUCE KITCHEN CABINETS ARE SUPPLIED PER PLAN IN KITCHEN AND VANITY CABINET IN BATHROOMS. A KITCHEN RANGE HOOD (TYPE WITHOUT VENT) IS SUPPLIED. A MEDICINE CABINET WILL BE SUPPLIED.

PAINT:	PAINT IS SUPPLIED FOR ALL WALLS, INTERIOR PARTITIONS (WHITE), CEILING AREA (WHITE) AND TRIM (WHITE) FOR FINISHED AREAS. EXTERIOR PAINT MATERIAL IS SUPPLIED FOR DECKS AND STEPS. (TWO COATS FOR EACH AREA).

FLOORING:	QUALITY CARPET AND UNDERLAY IS SUPPLIED FOR THE LIVING ROOM, BEDROOMS AND HALLS. VINYL FLOORING IS SUPPLIED FOR KITCHENS, DINING AREA, FOYERS. BATHROOMS.

WITNESS:___________________________	BUYER:___________________________

WITNESS:___________________________	BUYER:___________________________

GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLIES LIMITED

JUNE 1, 2021

PAGE 3 OF 3

CUSTOM HOME PACKAGE INSTALLATION

(STAGE 3)-PAGE 1, 2 & 3 OF SPECIFICATIONS)-CUSTOM HOME INSTALLATION

FOUNDATION:	INSTALLATION INCLUDES EXCAVATION, BACKFILL, INSULATED CONCRETE FOUNDATION WALL PER PLAN, 6" GRAVEL UNDER FLOOR, CONCRETE FINISHED FLOOR (IN MODELS WITH FULL BASEMENT), DAMPPROOFING, DRAIN TILE & GRAVEL (SITE IS ROUGH GRADED). EXCLUDES BLASTING, REMOVAL OF ROCK OR FILL OR THE SUPPLY OF FILL MATERIAL.

HOME PACKAGE:	INSTALLATION INCLUDED SILL PLATE, BEAM OR BEARING PARTITION, FLOOR JOISTS, FLOOR SHEATHING, BRIDGING, EXTERIOR WALLS INSTALLATION. PLACING ROOF TRUSSES, ROOF SHEATHING, ROOF SHINGLES AND ACCESSORIES. INSTALLING INTERIOR PARTITIONS, EXTERIOR DOORS AND WINDOWS, SIDING, SOFFIT, EAVESTROUGH AND DOWNSPOUTS. INSTALLING INSULATION AND DRYWALL, CRACKFILLING DRYWALL, EXTERIOR DOOR AND WINDOW TRIM. INSTALLING UNDERLAY, INTERIOR DOORS ANDHARDWARE, CLOSET RODS AND SHELVES AND BASEBOARD. PAINTING CEILINGS, WALLS, AND TRIM (2 COATS-WHITE). INSTALLING VINLY FLOORING, CARPET AND UNDERLAY. INSTALLING KITCHEN CABINETS AND BATHROOM VANITY AND MEDICINE CABINET (WHERE APPLICABLE). INSTALLING BATH AND KITCHEN FANS (RANGE HOOD), BATHROOM ACCESSORIES (SHOWER ROD, TOWEL BAR, TOILET TISSUE HOLDER AND SOAP & GRAB).

STEPS AND DECK: 	SUPPLY AND INSTALLATION OF FRONT AND REAR STEPS AND DECKS (WHERE APPLICABLE).

ELECTRIC/ELECT HEAT:	INSTALLATION OF 200 AMP SERVICE, PLUGS, SWITCHES, KITCHEN RANGE AND ELECTRIC DRYER OUTLETS, EXTERIOR WALL OUTLET. FIXTURE ALLOWANCE $500.00. INSTALLING ELECT HEAT WIRING AND HEATERS.

PLUMBING: INSTALLING 	COMPLETE PLUMBING SYSTEM, ONE PIECE BATH (TOPLESS) QUALITY FIXTURES WITH A  $500.00 FIXTURE ALLOWANCE. PLASTIC PIPE SUPPLY ANDABS DRAINAGE PIPE. A 40 GAL ELECTRIC HOT WATER TANK IS INSTALLED.

AIR EXCHANGER:	INSTALLATION OF A COMPLETE AIR-TO-AIR EXCHANGE SYSTEM.

WALKS/DRIVEWAY:	INSTALLATION OF GRAVEL WALKS AND DRIVEWAY (MAXIMUM 30'). (CULVERT, FILL, GRAVEL FOR CULVERT, REMOVAL OF ROCK, FILL OR BRUSH ARE NOT INCLUDED).

WITNESS:___________________________	BUYER:___________________________

WITNESS:___________________________	BUYER:___________________________