GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLY LIMITED

SPECIFICATION-SUMMER COTTAGES

JUNE 1, 2021

PAGE 1 OF 2

MATERIAL SUPPLIED IN SUMMER COTTAGE SHELL PACKAGE

(STAGE 1)-PAGE 1 OF SPECIFICATIONS-COTTAGE SHELL PACKAGE

COTTAGE PLANS:	A COMPLETE SET OF PLANS SHOWING BLOCKING, SKIRTING, FLOOR LAYOUTS, ELEVATIONS, DETAIL ETC. ARE SUPPLIED.

BEAMS:	2" X 10" LAMINATED BEAM PER PLAN.

FLOOR JOISTS:	FLOOR JOISTS SUPPLIED ARE 8" X 8" OR 2" X 10" PER PLAN. 1" X 3" BRIDGING IS SUPPLIED. 5/8 OSB T & G FLOOR SHEATHING IS SUPPLIED

EXTERIOR WALLS:	EXTERIOR PRE-FABRICATED WALL PANELS WITH 2" X 6" STUDS @ 16" 0.C., 2" X 6" PLATES, TOP AND BOTTOM. THE DOUBLE TOP PLATE MATERIAL IS SUPPLIED.  15/32 OSB SHEATHING, PINE COVE SIDING IS SUPPLIED. (VERTICAL PINE SHIPLAP SIDING WITH STRAPPING IS AN OPTION AND STANDARD SIDING ON CRESCENT AND QUEENSLAND MODELS).

INTERIOR PARTITIONS:	2" X 4" PLATES, SINGLE BOTTOM, DOUBLE TOP. 2" X 4" STUDS AT 16" O.C. ARE SUPPLIED.

ROOF/CEILING FRAMING: ROOF TRUSSES ARE SUPPLIED FOR HIP AND GABLE ROOFS AT 24" O.C.. GABLE END TRUSSES ARE SUPPLIED. 1" X 4" STRAPPING IS SUPPLIED FOR CEILINGS. THE EVANGELINE MODEL IS SUPPLIED WITH 4" X   5" OR 3" X 5" RAFTERS.

ROOF SHEATHING:	15/32 OSB SHEATHING IS SUPPLIED. V-JOINT PINE IS SUPPLIED WITH THE EVANGELINE MODEL FOR CATHEDRAL CEILINGS.

ROOF SHINGLES:	15# FELT PAPER, WEATHERSHIELD MATERIAL, 25 YEAR ASPHALT SHINGLES AND ROOFING CEMENT ARE SUPPLIED.

EXTERIOR DOORS:	PRE-HUNG STEEL ENTRANCE DOORS WITH HARDWARE ARE SUPPLIED.

WINDOWS:	VINYL PICTURE WINDOWS PER PLAN, VERTICAL SINGLE HUNG VINYL OR HORIZONTAL SLIDING WINDOWS ARE SUPPLIED UNLESS OTHERWISE SPECIFIED IN SOME MODELS. UNITS ARE EQUIPPED WITH SEALED GLASS AND SCREENS. MUTTON BARS ARE SUPPLIED FOR FRONT ELEVATIONS.

FLASHING:	ZINC FLASHING IS SUPPLIED FOR ALL WINDOWS AND EXTERIOR DOORS.

SIDING:	NOVA SCOTIA PINE COVE SIDING IS SUPPLIED. THE SIDING IS FOR APPLICATION OVER 2" X  4" FRAMING AND BUILDING PAPER.

GABLE TRIM/SOFFIT: 	NOVA SCOTIA V-JOINT PINEIS SUPPLIED. THE SOFFIT SYSTEM IS THE OPEN TYPE.

INSULATION:	R-24 FIBERGLASS INSULATION IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS. R-50 IS FIBERGLASS INSULATION IS SUPPLIED FOR CEILING AREAS, R-32 IS SUPPLIED FOR FLOOR AREA. 6 MIL POLYETHELENE VAPOUR BARRIER AND ADHESIVES AND TAPE ARE SUPPLIED.

WALLBOARD:	1/2@ GYPSUM BOARD IS SUPPLIED FOR EXTERIOR WOOD FRAME WALLS, INTERIOR PARTITIONS, CEILINGS IN FINISHED AREAS. CRACKFILLING MATERIAL AND TAPES ARE SUPPLIED.

MAIN STAIRS:	2" X 10" STRINGERS, SPRUCE TREADS, PINE RISERS ARE SUPPLIED IN THE CRESCENT AND QUEENSLAND MODELS.

NAILS/HARDWARE: 	SUPPLIED TO INSTALL COTTAGE PACKAGE.  SELLER

WITNESS:___________________________		BUYER:___________________________

WITNESS:___________________________		BUYER:___________________________

GRACE FACTORY HOMES

A DIVISION OF OCEAN LUMBER & BUILDING SUPPLY LIMITED

SPECIFICATION-SUMMER COTTAGES

JUNE 1, 2021

PAGE 2 OF 2

ADDITIONAL MATERIALS SUPPLIED IN THE COTTAGE CUSTOM PACKAGE

(STAGE 2)-PAGES 1 & 2 OF SPECIFICATIONS-COTTAGE CUSTOM PACKAGE

BASEBOARD:	PINE BASEBOARD MATERIAL IS SUPPLIED.

WINDOW TRIM:	PINE WINDOW TRIM IS SUPPLIED.

EXTERIOR DOOR TRIM:	PINE EXTERIOR DOOR TRIM IS SUPPLIED.

INSIDE DOORS:	PINE SLAB DOORS ARE SUPPLIED. PINE TRIM IS SUPPLIED. BI-FOLD DOORS ARE SUPPLIED FOR CLOSET AREAS AND WHERE INDICATED ON THE PLANS. THE BI-FOLD DOOR OPENINGS ARE THE MODERN TYPE WITHOUT TRIM.

CLOSETS:	CLOSET RODS AND PINE SHELVING ARE SUPPLIED FOR ALL CLOTHES CLOSET.

UNDERLAY:	3/8” SPRUCE SELECT UNDERLAY IS SUPPLIED FOR APPLICATION OVER WOODEN SUB-FLOOR EXCEPT 10MM MIRANDA BOARD IS SUPPLIED FOR AREAS WHERE VINYL FLOORING WILL BE INSTALLED.

BATHROOM:	ACCESSORIES, SOAP AND GRAB, TOILET TISSUE HOLDER, SOAP AND GRAB AND SHOWER BAR ARE SUPPLIED. A MEDICINE CABINET IS SUPPLIED. A BATHROOM VANITY IS SUPPLIED.

KITCHEN:	PRE-BUILT MODULAR OAK KITCHEN CABINETS ARE SUPPLIED PER PLAN IN KITCHEN. AND VANITY CABINET IN BATHROOMS. A KITCHEN RANGE HOOD (TYPE WITHOUT VENT) IS SUPPLIED.

PAINT:	PAINT IS SUPPLIED FOR ALL WALLS, INTERIOR PARTITIONS (WHITE),  CEILING AREA (WHITE) AND TRIM (WHITE) FOR FINISHED AREAS. EXTERIOR PAINT IS SUPPLIED FOR EXTERIOR TRIM, DECKS AND STEPS. (TWO COATS FOR EACH AREA).

FLOORING:	QUALITY CARPET AND UNDERLAY IS SUPPLIED FOR THE LIVING ROOM, BEDROOMS AND HALLS. VINYL FLOORING IS SUPPLIED FOR KITCHENS, DINING, AND BATHROOMS.

WITNESS:___________________________		BUYER:___________________________

WITNESS:___________________________		BUYER:___________________________